Calcutta 400 Series Parts Comparo 95, 98, B
Generally, the B model Calcutta is a modified upgrade of the 98 model.

	Calcutta Parts Comparison
	A (95)
	A (98)
	B (05)
	Comments

	
	
	
	
	Highlight = SAME

	CLUTCH
	Same
	Same
	
	

	Thumb Bar
	1645
	1645  (obs)
	3366
	 Alt: BNT3468

	Guide X
	1644
	1644 
	3365
	 

	Guide I
	1642
	3364
	3364
	 Alt: BNT3467

	Bar
	1643
	1643
	1643
	

	Washer
	1245
	1245
	1245
	 TLD 0210 Thin Shim

	E Clip
	425
	425
	425
	

	
	
	
	
	

	WORMSHAFT ASSEMBLY
	1632
	
	
	 All parts same?

	Guide Bar
	1634
	1634
	1634
	

	Shroud
	1633
	1633
	1633
	

	Worm
	1632
	1700
	1700
	

	
	
	
	
	

	Gear
	1632
	1703
	1703
	

	Washer
	1632
	1702
	1702
	

	Bush
	1632
	1243
	1243
	

	
	
	
	
	

	Bush
	1632
	1699
	1699
	

	Washer
	1632
	1838
	1838
	

	E Clip
	1632
	133
	133
	

	
	
	
	
	

	LINE GUIDE Assembly
	
	Same
	
	

	Guide
	1240
	1240
	3107
	Same but black?

	Pawl Cap
	1239
	1239
	0151
	

	Pawl
	42
	42
	42
	

	Washer
	60
	60
	60
	

	
	
	
	
	

	RHS 
	
	
	
	

	Right Side Plate
	1617
	2042
	3358
	

	Side Plate Bolt
	1615
	1698
	1698
	

	
	
	
	
	

	Handle Assembly
	1612
	1694
	3356
	

	Handle Nut Cap
	1612
	1692
	1663
	 Same?

	
	
	
	
	

	Cast Control Cap
	1614
	2098
	2098
	

	Cast Control Spacer (A)
	1614
	0009
	0009
	

	Cast Control Spacer (B)
	1614
	1696
	1696
	

	Spool Tension Spring
	1614
	1697
	1697
	

	
	
	
	
	Highlight = SAME

	Calcutta Parts Comparison
	A (95)
	A (98)
	B (05)
	Comments


	Calcutta Parts Comparison
	A (95)
	A (98)
	B (05)
	Comments

	
	
	
	
	Highlight = SAME

	Drive Assembly
	
	
	
	

	Drive Shaft
	1625
	1857


	3442
	Same (eBay)

Alt BNT 2435 ? (EyeD)

	Drive Shaft Bush
	
	0819
	0819
	

	Washers:  Plastic   

Copper
	1494
1230
	1494

1244
	1494

1476
	

	Handle Nut
	1612
	1693
	3441
	

	A/R Core
	1206 (obs)

13mm chrome
	= 2242 X
14mm grooved
	2926
13mm black
	2926 also= CT200B

	A/R Bearing
	1207 (obs)
	1207 (obs)
= 2925
	= RHS Plate  2925 Alt4447
	= KOYO EWC1012

 (+ many BCs)   

	(Right Side Plate+ Bearing)
	1617
	2042
	3358
	 = KOYO EWC1012

	Maingear
	1623
	1855
	3359
	

	Pinion
	1626
	1626
	3360
	

	Yoke
	1627
	1627
	1627
	

	DRAG
	
	
	
	

	Star Drag
	1573
	2002
	3357
	

	Star Drag Spacer Washer
	-
	1298
	2473
	 Same?

	Bellville Washers
	Set

1613
	1613 Light
	1613 Light
	

	
	
	1455 Heavy
	1455 Heavy
	

	
	
	
	
	

	Key Washer (A)
	1618
	1618
	1618
	Top

	Key Washer (B)
	1622
	1853
	1853
	

	Eared Washer
	1620
	1620
	1620
	

	
	
	
	
	

	Drag Washer (C)
	1619
	2061
	2061
	Top

	Drag Washer (B)
	1621
	2062
	2062
	Maingear

	Drag Washer (A)
	1624
	2063
	2063
	Under Maingear

	
	
	
	
	

	PARTS ABOVE Number 1622
	
	
	
	

	
	
	
	
	

	FRAME
	1640
	1640
	3367
	

	
	
	
	
	

	Left Side Plate
	1661
	1661
	3368
	

	
	
	
	
	

	Brake Collar
	1650
	1650
	1650
	

	Idle Gear Washer
	1647
	1647
	1647
	

	Idle Gear
	1648
	1648
	1648
	

	
	
	
	
	

	
	
	
	
	

	Click Plate
	1653
	1704
	1704
	

	Click Spring
	1655
	1705
	1705
	

	Click Switch Spring
	1657
	1706
	1706
	

	
	
	
	
	Highlight = SAME

	Calcutta Parts Comparison
	A (95)
	A (98)
	B (05)
	Comments


1995  (Made in Japan – ID = no O-ring under Cast Control Cap)
BNT0032 – BNT1662 (unique),  Entire Worm Drive Assembly is one part.

